| REQUIREMENTS OF PHOTO LOGGING - Scope of the Study The photo logging focuses on current condition of the well such as its diameter, casing and screens condition, a detailed description of any damages, and finally other relevant information and any other observation which can possibly impact this intervention. The Contractor shall liaise with authority concerned (NWSA) and ensure their knowledge of project The Contractor shall conduct site visits, perform the required survey, collect the required information and report it in the required format, in order to achieve the goal of this Contract. - Summary of Tasks - Assess the current condition of the existing borehole - Remove the well cover and replace it on completion of the assessment - Using a borehole camera and any other required equipment, determine the required technical data, including the exact total length of the water column in the borehole, the location, depth and status of perforated casing (filters or screens). - Provide photos showing all the information above - Submit a final report including the photos with GPS coordinates of the well and parts of casing and/or screens required to be replaced if any. - Format and Content of the Report The final report should be supported by technical documents, as follows: - Introduction with Name of the Contractor, Date, Location, Names and Contacts of people met for developing this study - Comprehensive description of context, background and development of the borehole - Technical description and status of the borehole and relevant facilities in general including photos and drawings showing the borehole construction, casing and screens as observed - Consultant’s recommendation for possibility to move to next step of intervention in this borehole, and justification for this intervention. - Submission of the Study Report - The final report should be submitted before the end of the Contract - Hard copy should be submitted in English, printed in color, clearly on A4 paper size, in 2 copies. - A soft copy of the report should be submitted at the same time with the hard copy. | ### متطلبات تسجيل وتصوير البئر #### 1) نطاق الدراسة يركز تصوير البئر على الحالة الحالية للبئر مثل قطرها، حالة الغلاف والفلاتر، ووصف مفصل لأي أضرار، وأي معلومات أخرى ذات الصلة، وأي ملاحظات أخرى يمكن أن تؤثر على هذا التدخل. يجب على المقاول التنسيق مع الجهة المعنية (إدارة مشاريع المياه) وضمان اطلاعهم على التنفيذ. يجب على المقاول إجراء زيارات الموقع، وتنفيذ المسح المطلوب، وجمع المعلومات المطلوبة وتقديمها بالشكل المطلوب لتحقيق هدف هذا العقد. #### 2) ملخص المهام - تقييم الحالة الحالية للبئر القائم - إزالة غطاء البئر واستبداله عند اكتمال التقييم - استخدام كاميرا البئر وأي معدات أخرى مطلوبة لتحديد البيانات الفنية المطلوبة، بما في ذلك الطول الكلي العمودي لعمود الماء في البئر، وموقع وعمق وحالة الغلاف المثقوب (المرشحات أو الشاشات) - تقديم صور تظهر جميع المعلومات المذكورة أعلاه - تقديم تقرير نهائي يشمل الصور مع الإحداثيات الجغرافية للبئر وأجزاء الغلاف و/أو الشاشات التي تتطلب استبدالها إن وجدت. #### 3) تنسيق ومحتوى التقرير يجب دعم التقرير النهائي بالوثائق الفنية على النحو التالي: - مقدمة تحتوي على اسم المقاول، التاريخ، الموقع، أسماء وجهات الاتصال للأشخاص الذين تم لقاؤهم لتطوير هذه الدراسة - وصف شامل للسياق والخلفية وتطوير البئر - وصف فني وحالة البئر والمرافق ذات الصلة بشكل عام بما في ذلك الصور والرسومات التي تظهر بناء البئر، الغلاف والشبكات كما هو ملاحظ - توصية الاستشاري بإمكانية الانتقال إلى الخطوة التالية من التدخل في هذا البئر، وتبرير هذا التدخل #### 4) تقديم تقرير الدراسة - يجب تقديم التقرير النهائي قبل نهاية العقد - يجب تقديم نسخة ورقية باللغة الإنجليزية، مطبوعة بالألوان، وبوضوح على حجم ورق A4، في نسختين - يجب تقديم نسخة إلكترونية من التقرير في نفس الوقت مع النسخة الورقية |
| --- | --- |
| REQUIREMENTS OF PUMPING TEST Unless specifically requested otherwise in BOQS , the following guidelines are to be followed: - Equipment Required : The following is a list of equipment usually required. The contractor should ensure all equipment is well maintained and in good condition. The contractor should consider having a backup for some items (such as dipper). - A well maintained pump, generator and ancillary equipment adequately sized for the purpose. - Rising pipes - Valve to vary flow (if not other means) - Adequate fuel or reliable power supply for the duration of the test. - Water tank of know capacity and/or a calibrated flow meter. - Stopwatch. - Dipper or pressure transducer. - Step Test: A Step Test or Step drawdown test is designed to establish the short term relationship between yield and drawdown for a borehole. The preferred method of testing is, - Start with the lowest pumping rate - There should be four or five steps - Each pumping rate to be equally spread between steps (e.g. 1 l/s, 2l/s, 3l/s, 4l/s etc) - All steps to be of equal length. Between 60 and 120 minutes preferred. The step duration may be increased or decreased during the first step depending on if the water level is continuing to fall - The final step pumping rate should be at or higher than the intended working pumping rate. - Constant Rate Test A constant rate test provides data to determine the transmissivity of an aquifer. The duration of a constant flow test should be long enough for the water level to reach or at least approach equilibrium. For tests carried out, the duration should be 72 hours, unless a different duration is specified in BOQs section of the contract. The following procedures should be undertaken, - Measure water level in the borehole every 30 second for the first 10 minutes, then every minute for the next 20 minutes and then every 5 minutes until 2 hours have elapsed. After 2 hours a frequency of measurements will be selected based on the drawdown rates so far (Please refer to the constant rate-test data form, enclosure 4) - The flow rate should be checked every 15 minutes for the first few hours of the test. It can be checked less frequently subsequently. If the flow rate varies greater than more than 10%, the pump or valves should be adjusted to bring back to the required rate. - Recovery Test A recovery test is used to collaborate information used from other pumping tests. It should take place after the constant rate test. The following should be considered when undertaking the recovery test, - As for the constant flow test, measure water level in the borehole every 30 seconds for the first 10 minutes, then every minute for the 20 minutes and then every 5 minutes until 2 hours have elapsed. After 2 hours a frequency of measurements will be selected based on the recovery rates so far ( Please refer to the Recovery rate-test data form, enclosure 4 ) - The pumps should not be removed from the borehole during the recovery test. - The recovery test should be maintained until the water level has recover - The organization representative will decide when the water level has recovered substantially enough to terminate the test. | ### متطلبات اختبار الضخ: ### ما لم يُطلب خلاف ذلك في جداول الكميات (BOQ)، يجب اتباع الإرشادات التالية: - المعدات المطلوبة القائمة التالية عبارة عن المعدات التي غالبا ما تطلب , يجب على المقاول التأكد من أن كل المعدات المطلوبة بحالة جيدة. و يجب على المقاول أيضا الأخذ بنظر الإعتبار إمتلاكه بعض المستلزمات ) مثل شريط قياس الهبوط) - مضخة أبار, مولد بحالة جيدة و بقية المستلزمات المساعدة و بالحجم الملائم للغرض - الأنابيب العمودية الرافعة - محبس التحكم بمعدل الضخ ) في حالة عدم تواجد المحبس ممكن استخدام اي وسائل اخرى - الوقود الكافي أو أي مصدر موثوق لتوليد الطاقة لضمان استمرارية الاختبار - خزان مائي ذو سعة معروفة و/ أو عداد مائي معاير - ساعة توقيف - متر قياس التغيير في المنسوب )الشريط الغاطس ( أو قياس المنسوب عن طريق الضغط. - خطوات الاختبار: الاختبار التدريجي أو التدرج في هبوط المنسوب يصمم بفترات قصيرة بغرض دراسة العلاقة بين التدفق و الهبوط في المنسوب في البئر و ويفضل عمل الاختبار بالطريقة التالية : - تبدأ عملية الضخ من معدل التدفق الأدنى. - يجب أن تكون الخطوات أو التدرجات في رفع المعدل أربع أو خمس مراحل - كل معدل تدفق تكون الزيادة بقدر متساوي في كل الخطوات ) مثلا 1 لتر/ث , 2 لتر/ث , 3 لتر/ث , 4 لتر/ث). - الفترة الزمنية لكل الخطوات متساوية المدة، و يفضل أن تكون ما بين 60 الى 120 دقيقة. مدة الخطوة ربما تزيد أو تقل خلال الخطوة الأولى في حالة تواصل هبوط مستوى الماء. - يجب أن يكون معدل الضخ في الخطوة الأخيرة عند أو أعلى من معدل الضخ التشغيلي المقصود. 3)اختبار الضخ المستمر اختبار الضخ المستمر يعطي معلومات عن النفاذية للطبقات الجوفية. المدة المستخدمة للاختبار يجب أن تكون كافية لمنسوب الماء حتى يصل أو على الأقل حتى يتوازن. حيث هذه الاختبارات تعمل لمدة 72 ساعة, مالم تحدد مدة أخرى في جداول الكميات . والإجراءات التالية يجب أن تؤخذ بعين الاعتبار: - قياس منسوب الماء في البئر كل 30 ثانية في العشر الدقائق الأولى، بعد ذلك أخذ المنسوب كل دقيقة للعشرين دقيقة الأخرى وبعدها أخذها كل خمس دقائق وحتى انقضاء الساعتين. بعد ساعتين من تواتر القياسات ستؤخذ القياسات بناء على الهبوط في المنسوب الآني) يرجى العودة الى استمارة اختبار التدفق الثابت، مرفق رقم 4) - المعدل في التدفق يجب أن بفحص كل 15 دقيقة في الساعات الأولى للاختبار، بالإمكان بعدها أن تفحص في فترات متباعدة، إذا تغير معدل التدفق بنسبة تزيد عن 10%، ويجب تعديل محبس المضخة للعودة للمعدل الصحيح. - اختبار عودة المنسوب ويستعمل في معايرة المعلومات القادمة من الاختبارات السابقة، ويجب أن تؤخذ بعد اختبار الضخ المستمر، وعند عمل الاختبار يجب مراعاة ما يلي: - كما في اختبار الضخ المستمر قياس مستوى الماء يكون كل 30 ثانية من العشر الدقائق الأولى، وبعدها يتم القياس كل دقيقة من العشرين دقيقة التالة وبعدها كل خمس دقائق وحتى انقضاء الساعتين. بعد ساعتين من تواتر القياسات ستؤخذ القياسات بناء على العودة في المنسوب الآتي) يرجى العودة الى استمارة اختبار عودة المنسوب، مرفق رقم 4) - يجب عدم إزالة المضخة من البئر في هذه المرحلة - يجب الاستمرار في اختبارات عودة المنسوب الى حين عودة المنسوب كاملا. - المهندس الممثل للمنظمة سيقرر ما هو المنسوب المناسب لعودة المنسوب لإنهاء الاختبار |
| REQUIRMENTS OF CHEMICAL, PHYSICAL, AND BACTERIOLOGICAL TEST: Sample Collection: - Utilize sterile sample bags or containers specifically designed for water testing. - If the water is chlorinated, neutralize the chlorine using an appropriate method to prevent contamination or alteration of sample composition. - Thoroughly sanitize the sampling site. Water Analysis: - Physical Analysis: - Analyze for total solids, turbidity, conductivity, pH, and other relevant parameters. - Chemical Analysis: - Conduct a comprehensive chemical analysis, including total alkalinity, hardness, essential cations and anions, and other parameters as per WHO and Yemeni standards. - Employ accredited laboratory equipment and procedures. - Bacteriological Analysis: - Perform bacteriological analysis using approved laboratory equipment and methods. - Conduct at least two bacterial cultures: one for total coliform bacteria and one for fecal coliform bacteria. - Utilize appropriate nutrient media and filters for each culture. - Incubate the cultures for 24 hours in suitable incubators according to testing requirements. Field Analysis: - If the laboratory is far from the sampling location, conduct field analysis for certain parameters as recommended. Timeliness: - Adhere to WHO and Yemeni standards for the timeframe within which physical, chemical, and bacteriological tests must be completed. Sample Transportation and Storage: - Transport and store samples to the laboratory using appropriate containers and incubators. Test Report: - Prepare a comprehensive water testing report encompassing physical, chemical, and biological analyses. - The report must be signed and stamped by the accredited laboratory that conducted the tests. - Include a summary of the results, indicating the potability of the water, recommendations, and necessary proposals. - Ensure the report complies with technical procedures and follows the instructions of the supervising engineer. | متطلبات الاختبارات الكيميائية والفيزيائي والبيلوجي: أخذ العينات: - يتم أخذ العينات من المياه المراد اختبارها بواسطة أكياس أو عبوات للعينات معقمة خاصة بذلك. - في حال كانت المياه مكلورة، تتم إزالة حبوب إبطال مفعول الكلور بطريقة مناسبة تضمن عدم تلوث أو تغير في تركيب العينات. - يتم تعقيم مكان أخذ العينة بشكلٍ دقيق. تحليل المياه: - التحليل الفيزيائي: - يتم تحليل كل من المواد الصلبة، العكورة، الموصلية، الأس الهيدروجيني، وغيرها من المعايير ذات الصلة. - التحليل الكيميائي: - يتم إجراء تحليل كيميائي شامل يشمل القلوية الكلية، العسرة، الكاتيونات والانيونات الأساسية، وغيرها من المعايير وفقًا لمعايير منظمة الصحة العالمية والمواصفات اليمنية. - يتم استخدام أجهزة وطرق عمل مخبرية معتمدة. - التحليل البكتيريولوجي: - يتم إجراء التحليل البكتيريولوجي باستخدام أجهزة وطرق مخبرية معتمدة. - يتم إجراء مزرعتين على الأقل للبكتريا القولونية الكلية ومزرعة للبكتيريا القولونية البرازية باستخدام وسط غذائي ومرشحات مختلفة لكل منهما. - يتم حضانتها لمدة 24 ساعة في حضانات مناسبة حسب احتياجات الفحص. الاختبارات الحقلية: - إذا كان المختبر بعيدًا عن مصدر أخذ العينة، يُنصح بإجراء تحليل ميداني لبعض المعايير. المدة الزمنية: - يتم الالتزام بمعايير منظمة الصحة العالمية والمواصفات اليمنية فيما يتعلق بالمدة الزمنية التي يجب خلالها إكمال الاختبارات الفيزيائية والكيميائية والبكتيريولوجية. نقل العينات وحفظها: - يتم نقل العينات إلى المختبر وتخزينها باستخدام حاويات وحاضنات مناسبة. تقرير الاختبار: - يتم تحضير تقرير شامل لاختبارات المياه يشمل التحاليل الفيزيائية والكيميائية والبكتيريولوجية. - يجب أن يكون التقرير موقعًا ومختومًا من قبل المختبر المعتمد الذي أجرى الاختبارات. - عناصر الاختبار تكون بحس الاستمارة المرفقة. - يتضمن التقرير ملخصًا للنتائج، مع تحديد صلاحية المياه للشرب أو عدم صلاحيتها، والتوصيات والمقترحات اللازمة، وكل ما يلزم لإنهاء البند وفقًا للأصول الفنية المتبعة وتعليمات المهندس المشرف. |