| MSF OCB – YEMEN- Lahj Gove, AL Farsha PHCC. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bill Of Quantities for Fabrication and Construction Safety Box Reducer. |  |  |  |  |  |
| All work shall be complete and ready for use in the correct way. The works are described in technical specifications, drawings, and including design, materials, and installation way. |  |  |  |  |  |
| No | Item description | UNIT | Q.t | Unit price | Total Price |
|  | Manufacture, Supply, and installation of the Safety Box Reducer body is made of metal plate 8mm thick with dimensions of W= 32cm, L= 50cm, and H= 100cm. The work includes the following specifications: - The Top frame with the loading door and chimney spigot - Grill: made of reinforcement bar 1@10cm with Ø 14mm. - All corners are supported by metal angle 2inch and 4mm thick. - Access door for SBR 30X30cm. Chimney: - Steel galvanized pipe: Ø 150 mm ideally, min. 4 mm thick, at least 4m long. - Chimney cap for protection. - Cables, tensioning devices, and accessories. - All drawings and layouts are attached. The contractor shall provide a sample of the materials before starting the manufacture of the safety box reducer. The work shall include all required accessories and components, as well as the application of primer paint to the entire body. All tasks must be completed in accordance with the specifications, attached drawings, and the instructions of the supervising MSF Engineer. | LS | 1 |  |  |
|  | تصنيع وتوريد وتركيب هيكل مخفض صندوق الأمان مصنوع من صفيحة معدنية بسمك 8 مم، بأبعاد عرض 32 سم، طول 50 سم، وارتفاع 100 سم. يشمل العمل المواصفات التالية: 1. الإطار العلوي مع باب التحميل وصنبور المدخنة. 2. الشبكة: مصنوعة من قضيب تسليح بسمك 10 سم، وقطر 14 مم. 3. جميع الزوايا مدعومة بزاوية معدنية بسمك 2 بوصة و4 مم. المدخنة: 5. أنبوب فولاذي مجلفن: قطر مثالي 150 مم، بسمك 4 مم كحد أدنى، وطول 4 أمتار على الأقل. 6. غطاء حماية للمدخنة. 7. الكابلات، وأجهزة الشد، والملحقات. 8. جميع الرسومات والمخططات مرفقة. على المقاول تقديم عينة من المواد قبل البدء في تصنيع مخفض صندوق الأمان. يشمل العمل جميع الملحقات والمكونات اللازمة، بالإضافة إلى طلاء الأساس على كامل الهيكل. يجب إنجاز جميع الأعمال وفقًا للمواصفات والرسومات المرفقة وتعليمات المهندس المشرف. |  |  |  |  |
|  | The Total cost |  |  |  |  |

| Bill of Quantities for Fabrication and Construction Glass Crusher Machine |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| All work shall be complete and ready for use in the correct way. The works are described in technical specifications, drawings, and including design, materials, and installation way. |  |  |  |  |  |
| No | Item description | UNIT | Q.t | Unit price | Total Price |
|  | Manufacture, Supply, and installation of glass crusher made of metal plate with dimensions of W= 20cm, L= 20cm, and H= 100cm. The work includes the following specifications: - The body sides are made of metal plate 4mm thick. - All corners are supported by metal Angle 2inch and 4mm thick. - The shutter base dimensions 16.5x16.5cm and 8mm thick. - The hammer dimensions (W=15cm, L=15cm, H=34cm) must be filled by compacted sand to weigh 15 kg, the mold of the hammer is made of metal plate 4mm thick. - Wired sling to hold the hammer with Ø 8 mm. The work shall include all required accessories and components, as well as the application of primer paint to the entire body. All tasks must be completed in accordance with the specifications, attached drawings, and the instructions of the supervising MSF Engineer. | LS | 1 |  |  |
|  | تصنيع وتوريد وتركيب كسارة زجاج من صفائح معدنية بأبعاد العرض = 20 سم، الطول = 20 سم، والارتفاع = 100 سم. يشمل العمل المواصفات التالية: 1- جوانب الهيكل مصنوعة من صفائح معدنية بسمك 4 مم. 2- جميع الزوايا مدعومة بشلمنات معدنية 2 انش وسمك 4 مم. 3- أبعاد قاعدة الكسر المتحركة 16.5 × 16.5 سم وسمك 8 مم. 4- أبعاد المطرقة (15 عرض × 15 طول × 34 ارتفاع) سم، تُملأ بالرمل المضغوط بوزن 15 كجم. قالب المطرقة مصنوع من صفائح معدنية بسمك 4 مم. 5- حبل سلكي لحمل المطرقة بقطر 8 مم. يشمل العمل جميع الملحقات والقطع اللازمة، بالإضافة إلى طلاء بريمر لهيكل الكسارة بالكامل، وجميع ما يلزم لإتمام العمل وفقًا للمواصفات والرسومات المرفقة وتعليمات المهندس المشرف. |  |  |  |  |
|  | The Total cost |  |  |  |  |