United Nations Population Fund,

UNFPA Yemen

Haddah St. behind Lazourde Hotel

Sana’a, Yemen

Tel: +967 1 433160

Website: www.unfpa.org

Email:

procurement.yemen@unfpa.org

Date: 18/12/2024

Request for Quotation No. YEM/2024/009

Provision of Prefab Offices for Aden

Dear Sir/Madam,

We hereby solicit your quotations for the Provision of Prefab Offices for Aden premises as below specifications:

| Technical specifications of the Prefab Office |  |  |
| --- | --- | --- |
| FLOOR | System | Specially Shaped profile and bolted joint construction in use. |
|  | Floor coating | 2 mm thick Wooden Patterned PVC vinyl. |
| WALLS | System | Specially shaped sandwich panel with Polyurethane 32kg/m3 insulation. |
|  | Outer Surface Covering | Ral 9002 electrostatic painted galvanized steel sheet. |
|  | Inner Surface Covering | Ral 9002 electrostatic painted galvanized steel sheet. |
|  | Wall Isolation | Produced with 100 mm thick Polyurethane, full thermal insulation, healthy and safe materials. The tapered corner joints should be covered with 0.30 mm sheet steel. |
| ROOF | Roof System | Special shaped clamped (grooved seam) EPS sandwich panels. Rainwater shall be drained through special clamping system. |
|  | Roof Isation | Produced with 125 mm EPS, full thermal insulation, healthy and safe materials. |
|  | Outer Surface Coating | Ral 9002 electrostatic painted galvanized steel sheet. |
|  | Inner Surface Covering | Ral 9002 electrostatic painted galvanized steel sheet. |
| DOORS | External door | Metal door made of 1.2 mm metal frame and 0.5 mm steel sheet and leak-proof seal. Resistant to scratches. The door is insulated against blow, heat and sound with 20 kg/m³ density EPS. |
|  | Internal Door | 755x1980 mm American wood door with electrostatic painted galvanized metal frame |
| WINDOWS | Window | 890x1135 mm PVC window. Windows are single wing and opens 90 degree. |
|  | Sill | PVC is used on the window sill of the inside edge. |
|  | Glass | Double glazing glass |
| PAINTING | Exterior and Interior | Ral 9002 coat coil painted. |
|  | Metal Parts | Ral 1019 electrostatic painted steel sheet. |
| ELECTRIC INSTALLATIONS | System | On the plaster |
|  | Cable | For plugs 3x2,5 , for lighting 2x1,5 and for commutator 3x1,5 NYM TSE certified cables in use. |
|  | Lighting | Appropriate lighting bulbs |
|  | Plugs and Switches | Located in every room. |

Ground Floor

| Kitchen | 4.00 x 2.70 |
| --- | --- |
| Four office rooms | 4.00 x 4.80 |
| One office room | 7.00 x 2.70 |
| One office room | 3.00 x 5.00 |
| Stairs | 3.00 x 2.70 |
| Entrance | 3.00 x 5.00 |

First Floor

| Head office with WC | 7.00 x 3.70 |
| --- | --- |
| Meeting room | 10.00x 3.20 |
| Empty area with a door to connect a second building | 12.00 x 9.00 |

The services are to be delivered maximum in 6 weeks upon issuing of PO according to the location mentioned.  The quotation shall be valid at least for 3 months after the closing date.

If you are interested in submitting quotations for these items, kindly fill in the attached Quotation Form or your own form signed and stamped and make sure the quotations are submitted and send  by E-mail only to the address indicated below not later than 29/12/2024, 13:00 (Sana’a time):

Email: procurement.yemen@unfpa.org

Bidders must enter the following text in the email subject line:

“RFQ/YEM/2024/009– “Provision of Prefabs”

Please submit your quotation in US Dollars currency. Conversion of currency into the UNFPA preferred currency, if the offer is quoted differently from what is required, shall be based only on UN Operational Exchange Rate prevailing at the time of competition deadline.

Bidders requiring clarifications to the RFQ process and/or RFQ documents may be addressed in writing to:

Khalil Al-Hakimi , al-hakimi@unfpa.org

Bidders should NOT submit any Bid to this contact or your Bid will be declared invalid, as UNFPA will not able to guarantee the confidentiality of the Bidding process.

Bidders may request clarifications not later than 25th December 2024, 15:00 Sana’a time.

Quotations submitted by email must be free from any form of virus or corrupted contents, or the quotations shall be rejected.

It shall remain your responsibility to ensure that your quotation will reach the address above on or before the deadline. Quotations that are received by UNFPA after the deadline indicated above, for whatever reason, shall not be considered for evaluation. Kindly ensure that it is signed and in the PDF format, and free from any virus or corrupted files.

Please take note of the following requirements and conditions pertaining to the supply of the above mentioned goods:

Partial quotes -  not permitted

Partial delivery – not permitted

Warranty - The warranty period shall be minimum 12 months.

DOCUMENT CHECKLIST - Please remember to submit the following documents:

| No. | DOCUMENT NAME |  |
| --- | --- | --- |
| 1 | Annex 1 – Quotation Form |  |
| 2 | - Latest Business Registration Certificate; - Latest Internal Revenue Certificate / Tax Clearance; - Authorization of the Company as a Construction Agent |  |
| 3 | Written Self-Declaration of not being included in the UN Security Council 1267/1989 list, UN Procurement Division List or other UN Ineligibility List |  |

Bidders may modify their offers in writing prior to the submission deadline. The bidder must submit the proposed modification via email that must be clearly marked as “MODIFICATION”.

In this case the previous offer will be declined and the modified offer shall be considered for the evaluation process.  Also, if the same bidder has submitted several offers before the bid deadline superseding each other only the last received offer will be considered for opening and further evaluation.

Note: Current UNFPA supplier policies apply to this solicitation and can be found at: http://www.unfpa.org/suppliers.

Best regards,

UNFPA

Yemen

Quotation Form

Name of Bidder:

Date of Bid:

Request for Quotation No:

Currency of Bid price:

Delivery time (weeks from receipt of order till dispatch):

Expiration of Validity of Quotation (The quotation shall be

valid for a period of at least 3 months after the Closing date.):

Price Schedule:

| Item No. | FCA price/unit specify port of shipment | ## Quantity | Transportation cost to port of Destination (specify mode of trp.) | Shipment volume (cbm) and Weight of goods | Total DAP (Destination port) | Delivery schedule (months) |
| --- | --- | --- | --- | --- | --- | --- |

In your offer, please include:

- Specific technical specifications of products offered

- Quality standard of the products

Vendor’s Comments:

I hereby certify that this company, which I am duly authorized to sign for, accepts the terms and conditions of UNFPA (http://www.unfpa.org/resources/unfpa-general-conditions-contract ) and we will abide by this quotation until it expires.

Name and title								Date and Place